# Face Capture

A simple guide to the design

The idea. The person keeps their head still while you move the phone around their face. The app takes nine guided captures, followed by a slow sweep that collects more photos.

# What each step captures

Left and right mean the person’s own left and right. L ½ and R ½ mean a partial side view. The app calls the chin step “Jaw.”

| Step | Move the phone | What the view adds |
| --- | --- | --- |
| 1. Front | Directly in front | Center of the face; sets the starting reference. |
| 2. L ½ | Partway to their left | Left cheek and side of the nose. |
| 3. Left | Farther to their left | Left profile: nose, lips, chin, and jaw. |
| 4. R ½ | Partway to their right | Right cheek and side of the nose. |
| 5. Right | Farther to their right | Right profile: nose, lips, chin, and jaw. |
| 6. Brow | Above eye level, looking down | Forehead, brow, and upper nose. |
| 7. Jaw | Below the chin, looking up | Underside of the chin and lower jaw. |
| 8. Ear L | Farther around the left side | More coverage near the left ear and outer jaw. |
| 9. Ear R | Farther around the right side | More coverage near the right ear and outer jaw. |
| 10. Photo sweep | Slowly across nearby angles | Overlapping photos for the detailed 3D appearance. |

## Why return to the sides?

The ear views go slightly farther around the head to capture more side surfaces. They overlap with the earlier profiles, so full ear coverage is not guaranteed.

## What happens at each guided stop?

The app checks the phone’s position and waits for a short, steady hold. On a LiDAR phone, it normally combines eight depth frames into one depth map and keeps one color photo. Without LiDAR, it keeps a station photo only.

# The technology, in plain language

Rear camera: records appearance. It captures the face’s color and visible detail.

LiDAR: records depth. On supported phones, it measures distance to visible surfaces. Several views help build the face’s shape.

ARKit: tracks the phone. Apple’s tracking software estimates the phone’s position and direction as you move.

Vision: finds the face. Apple’s software locates the face and eyes for the guides and the starting head reference.

Object Capture: builds a model from photos. Apple’s software uses overlapping photos to build the detailed 3D display model.

# Two types of capture, two jobs

Guided stops: establish the shape. With LiDAR, these stops record depth plus color. The server aligns and combines the depth maps into the surface used for measurement.

Photo sweep: add overlapping views. The sweep saves color photos and camera positions for the display model. These saved photos contain no depth maps.

Saved camera calibration helps the software place images and depth in 3D.

# What happens when you finish?

Tap “Finish & save scan.” The app saves the capture on the phone.

Upload it from Sessions. The server processes the saved images and any depth.

View the 3D model. With depth, a separate surface supports the measurement pipeline.

# What to remember

Keep the head and expression still. After the front capture, the app assumes the head stays in the same place.

A phone without LiDAR can create a photo-based display model. The current system does not use it for volume measurements.

A detailed-looking model does not prove measurement accuracy. Real-face accuracy still needs validation.

Based on the current project code reviewed September 8, 2026: CaptureController.swift, RearCaptureBackend.swift, SessionWriter.swift, and server/processing.py. The installed phone build may differ.